Database for InVEST sediment and nutrient model parameters

Ailsa McCulloch and Perrine Hamel

Fall 2015

# Overview

The InVEST sediment and nutrient models require input parameters that are specific to each type of land use/land cover. This document describes the database compiled by the Natural Capital Project to select these parameters based on global peer-reviewed literature.

# Database description

Each entry to the database correspond to one data point (i.e. the value of at least one of the model parameters). Below is a description of each column.

## Nutrient parameters

- ID (unique row identifier)

- LULC

Land use land cover (LULC) type according to the IGBP classification, although some classes were lumped following the table below:

| Database class | IGBP class |
| --- | --- |
| Forest | Evergreen Needleleaf Forest |
|  | Evergreen Broadleaf Forest |
|  | Deciduous Needleleaf Forest |
|  | Deciduous Broadleaf Forest |
|  | Mixed Forest |
| Shrublands | Closed Shrublands |
|  | Open Shrublands |
| Woody Savannas | Woody Savannas |
|  | Savannas |
| Grasslands | Grasslands |
| Permanent Wetlands | Permanent Wetlands |
| Croplands | Croplands |
| Urban and Built-Up | Urban and Built-Up |
| Cropland/Natural Vegetation Mosaic | Cropland/Natural Vegetation Mosaic |
| Barren or Sparsely Vegetated | Snow and Ice |
|  | Barren or Sparsely Vegetated |
| Waterbodies | Water Bodies |
| -- | Interrupted Areas |
| -- | Missing Data |

All forest types have been aggregated into a broader “Forest” category. Information helpful for Forest sub-classification is given in “Other LULC Details”. Similarly, closedand open Shrubland, one one hand, and all types of Savannas, on the other hand, were lumped into broader categories; when available, additional information is given in “Specific LULC”.

- Other LULC Details

This is a temporary category containing information for the sub-classification of the Forest class, when available. (Ultimately, this information may be integrated to the LULC column.)

- Specific LULC

This column gives more specific information on LULC details where possible. Open and closed shrublands are lumped in the same LULC class, but when more information exists (as to whether the Shrubland is an Open or Closed Shrubland), the information was retained in this category. The same holds for Savannas; when available, information on whether the LULC may be “woody savannah” is provided in this section. For the Croplands LULC class, crop type/species were specified with percentage abundances, if given. For Urban and Built-Up LULC class, additional information on density or type of development is provided.

- Continent

The continent to which the study site belongs.

- Country

The country to which the study site belongs.

- State/Province

The state or province to which the study site belongs (if given).

- Specific Location

If a more detailed location is given (beyond state or province), this is noted. For example, a city/town may be listed, or a study forest.

- Köppen Climate

Major Koppen-Geiger climate category, i.e. one of:

  - Tropical/megathermal

  - Dry (arid and semi-arid)

  - Temperate/mesothermal

  - Continental/microthermal

  - Polar and alpine

We added this information by overlaying the specific location to a Köppen climate map. Therefore, there are many instances where a unique Köppen climate could not be assigned in the database: if a country was given but there are multiple Köppen climates associated with it, n/a was assigned.

- Nutrient form

N or P for Nitrogen- or Phosphorus-related parameter

- eff_n: retention efficiency for N reported in the study (often mean or median)

- eff_n_min: minimum retention efficiency for N, when reported

- eff_n_max: maximum retention efficiency for N, when reported

- eff_p: retention efficiency for P reported in the study (often mean or median)

- eff_p_min: maximum retention efficiency for P, when reported

- eff_p_max: maximum retention efficiency for P, when reported

- load_n: load (or export coefficient) for N (often mean or median)

- load_n_min: minimum load for N, when reported

- load_n_max: maximum load for N, when reported

- load_p: load (or export coefficient) for P (often mean or median)

- load_p_min: minimum load for P, when reported

- load_p_max: maximum load for N, when reported

- ret_len_n: distance over which N retention was calculated

- ret_len_p: distance over which P retention was calculated

- Proportion Subsurface: proportion of the load that is transported by subsurface flow

- Annual Precipitation*

- Elevation*

- Soil Type*

- Soil Drainage*

- Slope*

- Original Data Source: reference

- Modeled Data:

* Data have not been quality controlled

## Sediment parameters

- ID (unique identifier)

Currently there are 988 unique records in the sediment database.

- LULC

The same process was used in the sediment database as the nutrient database, defining LULC by IGBP class. See Section 2 under “Nutrient parameters” for more information.

- Other LULC details

The same process was used in the sediment database as the nutrient database here. See Section 3 under “Nutrient parameters”.

- Specific LULC

The same process was used in the sediment database as the nutrient database here. See Section 4 under “Nutrient parameters”.

- Continent

The continent to which the study site belongs.

- Country

The country to which the study site belongs.

- State or Province

The state or province to which the study site belongs (if given).

- Specific Location

If a more detailed location is given (beyond state or province), this is noted. For example, a city/town may be listed, or a study forest.

- Köppen Climate

The same process was used in the sediment database as the nutrient database here. See Section 9 under “Nutrient parameters”.

- Assumed Köppen Climate

In some cases, as noted in the “Köppen Climate” section, the given location was not discrete enough to provide a Köppen Climate for the record. In these cases, a the Köppen Climate was assumed by the most prevalent Köppen Climate classification for that area. For instance, there are Temperate/mesothermal, Continental/microthermal, and Dry (arid and semi-arid) climates in the state of Oregon, but the most prevalent climate is Temperate/mesothermal. Where the specific location in Oregon was not given, the “Assumed Köppen Climate” was thus listed as Temperate/mesothermal.

- sedret_eff: the retention efficiency reported in the study (often mean or median)

- sedret_eff_min: minimum retention efficiency, when reported

- sedred_eff_max: minimum retention efficiency, when reported

- rough_rank

Currently, the database does not include many records for this parameter.

- rough_rank_min (ditto)

- rough_rank_max (ditto)

- cover_rank

Currently, the database does not include many records for this parameter.

- cover_rank_min (ditto)

- cover_rank_max (ditto)

- usle_c: C factor for the Universal Soil Loss Equation

- usle_c_min: minimum C factor for the Universal Soil Loss Equation

- usle_c_max: minimum C factor for the Universal Soil Loss Equation

- usle_p: P factor for the Universal Soil Loss Equation

- usle_p_min: minimum P factor for the Universal Soil Loss Equation

- usle_p_max: maximum P factor for the Universal Soil Loss Equation

- Buffer Width*

- Area Ratio*

- Annual Precipitation*

- Elevation*

- Soil Type*

- Hydraulic Conductivity (m/s)*

- Soil Drainage*

- Slope*

- Original Data Source

- Modeled Data

* Data have not been quality controlled

# Summary statistics

## Nutrient parameters

- Global statistics

Figure 1. N and P Data Locations by Number of Data Points

Figure 2. Phosphorus records

Figure 3. Nitrogen records

- Statistics per continent

Figure 4. TN data per continent

Figure 5. TP data per continent

## Sediment parameters

- Global statistics

Figure 6. Sediment Retention Efficiency Location by Number of Data Points

Figure 7. C factor for the Universal Soil Loss Equation (usle_c): Number of Data Points by location

Figure 8. P factor for the  Universal Soil Loss Equation (usle_p): Number of Data Points by location

Figure 9. LULC Class Distribution by Continent

Figure 10. Usle_C, LULC Distribution by Continent

Figure 11. Usle_P, LULC Distribution by Continent

Figure 12. Sediment Retention Efficiency, LULC Distribution by Continent

# Next steps

- Quality assurance of the eff_x, and load_x (e.g. by plotting them by category and checking that outliers have not been erroneously reported)

- Quality assurance of the usle_c, and usle_p values (including adding the Wischmeier corrected data from Brad)

- From the summary statistics, determine if additional points should be added to the database (e.g. where there is less than 3 points for a continent).

- Complete the climate field (a lot of NAs are due to the location not specific enough: would need to go back to the study to identify the climate zone, cf. Field 8: “Koppen climate”)

- Complete the Forest sub-category (currently Forest types are aggregated, cf. Field 3: “Other LULC Details”)

- Sediment database slopes: the values in the slope column (Column AH) are inconsistently reported (sometimes degrees are reported, in other instances, percentages are reported).

- All the precipitation and soil related fields should be quality-controlled

See next steps